EVENT VISION SCREENING – FULL PROCEDURE

(Free-Screen Method – No Personal Data Entry, No Printer Required)

Supplies Needed

- Vision screening device

- Event multi-parent signature consent form

- Stickers and sunglasses (when available)

- QR-code referral postcards

1. Consent Collection

- Confirm the parent or guardian has signed the event consent form before screening.

Sample of Form:

2. Device Startup

- Press and release the Power button.

- Allow ~30 seconds for the Home Screen to appear.

- Ensure the device battery level is adequate for the event.

3. Select “Age Range” (Screen Without Typing Data)

To run screenings without entering personal information:

- From the Home Screen, select: Tools → Age Range

- Choose the appropriate age range for the child.

- This allows screening with age only — no name, DOB, or gender required.

- Additional details can be added later if needed, but for event screenings they are not used.

4. Begin the Screening

- Tap Start to begin the binocular or monocular test.

- Position yourself approximately 3 feet from the child.

- The device will use a fixation target and audio cues to keep the child’s attention.

- Hold the device steady until the reading completes.

- Results will appear automatically on the touchscreen.

5. Understanding the Results

- The device will display either:

  - PASS (no risk factors detected), or

  - REFER (one or more risk factors detected)

- Results can be printed or saved, but for event screenings:

  - You will not print, and

  - You will not manually save the record.

6. If the Child PASSES

- No postcard is given.

- When the device asks “Save this record?” → select NO.

- Return to the Home Screen to screen the next child.

- Important: Even when you select “NO,” the device still automatically stores the screening data in its internal History.

7. If the Child is FLAGGED

- Fill out a referral postcard for the parent.

Example of postcard

- Mark the specific condition(s) the child was flagged for.

- Hand the postcard to the parent and encourage them to scan the QR code for more details.

- When the device asks “Save this record?” → select NO.

- Return to the Home Screen to continue screening.

8. Continue Screening

- Repeat the process for each child.

- Do not enter names, dates of birth, or any personal identifiers into the device.

- Stickers or sunglasses may be given to all children to create a positive experience.

9. After the Event

- Power off the device and return it to its case. Be sure power cord is put into the screening case.