CHILD CARE VISION SCREENING – FULL PROCEDURE

(Free-Screen Method –No Printer Required)

Supplies Needed

- Vision screening device

- Stickers

- USB flash drive

1. Consent Collection

Prior to the screening, an MOU or parental consent form was provided to the facility.  Collect all paperwork.

2. Device Startup

- Press and release the Power button.

- Allow ~30 seconds for the Home Screen to appear.

- Ensure the device battery level is adequate for the event.

3. Select “Age Range” (Screen Without Typing Data)

To run screenings without entering personal information:

- From the Home Screen, select: Tools → Age Range

- Choose the appropriate age range for the child.

- This allows screening with age only — no name, DOB, or gender required.

- Additional details can be added later if needed, but for event screenings they are not used.

4. Begin the Screening

- Tap Start to begin the binocular or monocular test.

- Position yourself approximately 3 feet from the child.

- The device will use a fixation target and audio cues to keep the child’s attention.

- Hold the device steady until the reading completes.

- Results will appear automatically on the touchscreen.

5. Understanding the Results

- The device will display either:

  - PASS (no risk factors detected), or

  - REFER (one or more risk factors detected)

- Results can be printed or saved, but for event screenings:

  - You will not print, and

  - You will not manually save the record.

6. If the Child PASSES

- When the device asks “Save this record?” → select NO.

- Return to the Home Screen to screen the next child.

- Important: Even when you select “NO,” the device still automatically stores the screening data in its internal History.

7. If the Child is FLAGGED

- When the device asks “Save this record?” → select YES.

- Obtain the first and last name of the child and enter it into the device.

- Return to the Home Screen to continue screening.

8. Continue Screening

- Repeat the process for each child.

- Do not enter names, dates of birth, or any personal identifiers into the device unless the child is flagged in the screening.

- Stickers may be given to all children to create a positive experience.

9. After the Event

- Power off the device and return it to its case. Be sure power cord is put into the screening case.

- Be sure to collect all consent forms or MOU

10. Submit Materials to the SD Lions Foundation

- Provide the consent forms or MOU and the screening data to the SD Lions Foundation.

- The Foundation will:

  - Compile the screening data

  - Prepare and send a screening summary

  - Send the childcare facility the individual reports for children who were flagged for a full eye exam

If you are returning the device to the foundation, you do not need to download the data.

If you are keeping the device, the data needs to be sent to the foundation. Proceed to the next steps below.

Spot Vision Screener – Data Export Procedure (USB Method)

Purpose

To ensure all screening devices placed across South Dakota can reliably send screening data to the SD Lions Foundation, using a simple USB export process that requires no technical skill and works in all locations, including rural areas.

1. Required Materials

- Spot Vision Screener

- USB flash drive (provided in each screening kit)

- Access to email

Each kit should include a USB drive

2. When to Export Data

Data should be exported:

- After each screening event, before the device is returned or stored

3. Prepare the Device for Export

- Power on the Spot Vision Screener.

- Allow the Home Screen to fully load (about 30 seconds).

- Ensure the device battery is above 20% or plugged in.

4. Insert the USB Drive

- Plug the USB flash drive into the USB port on the bottom of the device.

- Wait 2–3 seconds for the device to recognize it.

5. Navigate to the Export Menu

On the device:

- Tap Tools

- Select Data

- Choose Export

6. Confirm Export Completion

- The screen will display a confirmation message when the export is complete.

- Remove the USB drive only after the device indicates it is safe to do so.

7. Send the Data to the SD Lions Foundation

Once the USB drive is removed:

Option A – Email (most common)

- Insert the USB drive into a computer.

- Open the exported file(s).

- Attach the file(s) to an email.

- Send to: jennie@sdlions.org (or your preferred address)

- Include the following in the email body:

  - Name/Location of screening event

  - Date(s) of screening

  - Club/group that did the screening

8. Verify Receipt (Foundation Step)

The SD Lions Foundation will:

- Confirm receipt of the data

- Import the file into the statewide database and contact the childcare facility with the summary and flagged kids.